UABAMS Cloud Interface and Data Structure Document

Server receive/process/store/send contract for Gateway, Cloud Dashboard, CSV reports, and Alert Map

| Prepared For | Prepared By | Date | Version |
| --- | --- | --- | --- |
| UABAMS review meeting | Sujata Jaiswal | 10 June 2026 | v1.0 |

Purpose: This document adds the missing technical details requested by management: what data the cloud server receives, what it validates and processes, what it stores, what it returns to the gateway, and which data types/structures are used.

# 1. Executive Summary

The UABAMS cloud module is the intermediate server-side application for gateway session ingestion and visualization. The gateway does not send individual sensor values as normal form data in the production flow. It closes a running session, creates a ZIP archive containing metadata, RMS, peak, fault and raw binary files, and uploads the ZIP to the cloud using HTTP PUT. The cloud stores the original archive, extracts and parses the files, writes parsed records into PostgreSQL, creates CSV downloads, and displays alert locations on a map.

| Manager Question | Answer in Current Implementation |
| --- | --- |
| What does the server receive? | A gateway session ZIP through PUT /api/v1/archive and optional real-time alert JSON through POST /api/v1/alerts. |
| What does the server process? | session_metadata.json first, then RMS 25 cm records, Peak 50 m records, Fault records and raw packet metadata. |
| What does the server store? | Original ZIP bytes, extracted binary files, archive/session metadata, parsed RMS/Peak/Fault/Raw records and alert events in PostgreSQL. |
| What does the server send to gateway? | HTTP response codes: 201/200 success, 200 duplicate accepted, 422 validation mismatch, 4xx/5xx error requiring retry. |
| What is shown in UI? | Cloud archive dashboard, upload form, CSV exports, parsed records, alert event table, browser notification and map marker for alert location. |

# 2. Source Documents Used

| Source | Key Requirement Used |
| --- | --- |
| TS of UABAMS.pdf | Spatial acceleration interval is 25 cm; system should transfer data to processing station and CRIS/TMS; alerts above 80 kmph with value and GPS location; data/reports stored in database or ASCII-compatible format. |
| interface_control_document.txt v1.2 | Exact gateway ZIP upload contract, file structure, binary record sizes, parser data types, alert JSON contract, cloud database model and API responses. |
| Current codebase | FastAPI endpoints, PostgreSQL tables, CSV export page, PyQt desktop upload UI and alert map implementation. |

# 3. End-to-End Workflow

Sensor nodes collect acceleration and encoder/GPS related information through gateway UART/RS-485 channels.

Gateway builds spatial RMS data around each encoder-derived 25 cm segment and peak records over 50 m windows.

When the train/session becomes idle, the gateway closes the session and writes session_metadata.json with sessionStatus = closed.

Gateway packages the complete session folder into one ZIP archive.

Gateway uploads the ZIP to cloud endpoint PUT /api/v1/archive.

Cloud validates filename, metadata, schema version, required files, fixed record sizes and duplicate key.

Cloud stores the ZIP and extracted binary files, parses binary records, saves PostgreSQL rows and generates CSV downloads.

If peak alert criteria are met, alert events are stored and shown in the Alert Map with browser notification support.

| Flow Area | Implementation Detail |
| --- | --- |
| Primary upload | Gateway -> Cloud: HTTP PUT /api/v1/archive with raw ZIP bytes. |
| Real-time alert | Gateway -> Cloud: HTTP POST /api/v1/alerts with JSON alert event. Current gateway firmware has this as planned/stub, but cloud endpoint is ready. |
| Cloud response | Cloud -> Gateway: HTTP status and JSON response. Gateway deletes local copy only after 200/201. |
| CSV/reporting | Cloud -> User: web page and download endpoints for CSV reports from PostgreSQL records. |

# 4. What the Server Receives

## 4.1 Archive Upload Request

| Item | Value / Data Type | Meaning |
| --- | --- | --- |
| Method | HTTP PUT | Gateway uploads one complete session archive. |
| Path | /api/v1/archive | Primary cloud ingestion endpoint. |
| Body | binary ZIP bytes | ZIP file containing all session files. |
| Filename | string | <gatewayId>__<trainId>__<sessionName>.zip |
| Content-Length | integer bytes | Provided by gateway; used for archive size. |
| Success response | HTTP 200 or 201 | Gateway can delete local ZIP/session folder. |
| Validation failure | HTTP 422 | Filename and metadata mismatch; gateway should retry/retain copy. |

| ZIP Path | File Type | Record Size | Cloud Action |
| --- | --- | --- | --- |
| session_metadata.json | JSON text | N/A | Parse first; validate gatewayId, trainId, sessionName, schemaVersion, sessionStatus. |
| rms/rms_25cm.bin | binary fixed records | 66 bytes | Parse spatial acceleration RMS records per about 25 cm segment. |
| peak/peak_50m.bin | binary fixed records | 302 bytes | Parse 50 m peak windows and alertGenerated flag. |
| faults/faults.bin | binary fixed records | 75 bytes | Parse gateway/sensor fault records. |
| raw/adxl_left.bin | binary length-prefixed packets | variable | Store raw packets and parse metadata for forensic replay. |
| raw/adxl_right.bin | binary length-prefixed packets | variable | Store raw packets and parse metadata for forensic replay. |
| raw/bogie.bin | binary length-prefixed packets | variable | Store raw packets and parse metadata for forensic replay. |
| raw/encoder.bin | binary length-prefixed packets | variable | Store raw packets and parse metadata for forensic replay. |

## 4.2 session_metadata.json Fields

| Field | JSON Type | Example | Cloud Use |
| --- | --- | --- | --- |
| schemaVersion | string | 1.0 | Reject/quarantine unsupported schema before binary parsing. |
| gatewayId | string | GW_BOGIE_001 | Logical gateway identity; part of archive/session key. |
| gatewaySerial | string | UABAMS-2026-0001 | Physical gateway serial, stored in gateway metadata. |
| trainId | string | TRAIN_07 | Train identifier; part of session identity. |
| firmwareVersion | string | 1.0.0 | Stored for traceability. |
| gatewaySoftware | string | UABAMS Gateway | Stored for traceability. |
| sessionName | string | SESSION_20260609_083015 | Unique session name; matched against filename. |
| sessionStatus | string | closed | Cloud treats active sessions as incomplete. |
| createdUtc | string ISO UTC | 2026-06-09T08:30:15Z | Session open time. |

# 5. Data Structures and Data Types

## 5.1 RMS 25 cm Record: rms/rms_25cm.bin

Purpose: One Rms25cmRecord per about 25 cm spatial segment. Record size is exactly 66 bytes, little-endian, fixed-size binary.

| Offset | Field | Type | Unit | Meaning |
| --- | --- | --- | --- | --- |
| 0 | masterCount | uint64_t | count | Encoder sync counter at segment end. |
| 8 | positionMm | int32_t | mm | Cumulative wheel position at segment end. |
| 12 | latitude | double | deg | WGS-84 latitude at segment midpoint. |
| 20 | longitude | double | deg | WGS-84 longitude. |
| 28 | gpsValid | bool/u8 | - | 1 means GPS fix valid; 0 means estimated/lost. |
| 29 | validMask | uint8_t | - | Bit 0 AL, Bit 1 AR, Bit 2 bogie data present. |
| 30-62 | al/ar/bg x/y/z mg | uint32_t x 9 | mg | RMS acceleration for ADXL Left, ADXL Right and Bogie axes. 0xFFFFFFFF means no data. |

RMS calculation: RMS values are computed from raw accelerometer samples collected during each spatial segment. ADXL conversion is 0.09765625 g/LSB; bogie conversion is 8.0/32768.0 g/LSB. The ADXL factor is flagged in gateway code as needing firmware confirmation.

## 5.2 Peak 50 m Record: peak/peak_50m.bin

Purpose: One PeakRecord50m per 50 m window. Record size is exactly 302 bytes, little-endian. It contains speed, alert flag and nine axis peak sub-records.

| Offset | Field | Type | Unit | Meaning |
| --- | --- | --- | --- | --- |
| 0 | windowStartMm | int32_t | mm | Track position at start of 50 m window. |
| 4 | windowEndMm | int32_t | mm | Track position at end of window. |
| 8 | speedKmph | float | km/h | Maximum speed observed within the window. |
| 12 | validMask | uint8_t | - | Union of available node data bits. |
| 13 | alertGenerated | bool/u8 | - | 1 if speed gate and axis threshold condition are met. |
| 14-301 | al_x...bg_z axis records | PeakAxisRecord x 9 | mg/mm/deg | Axis peak value, peak position, master count and GPS point. |

| PeakAxisRecord Field | Type | Unit | Meaning |
| --- | --- | --- | --- |
| peakValueMg | uint32_t | mg | Highest vibration peak value for the axis in the 50 m window. |
| peakPositionMm | int32_t | mm | Track position where the peak occurred. |
| peakMasterCount | uint64_t | count | Encoder master count where peak occurred. |
| peakLat | double | deg | Latitude used for alert map marker. |
| peakLon | double | deg | Longitude used for alert map marker. |

| Axis | Default Threshold mg | Node/Direction |
| --- | --- | --- |
| al_x | 10000 | ADXL Left X |
| al_y | 8000 | ADXL Left Y |
| al_z | 12000 | ADXL Left Z |
| ar_x | 10000 | ADXL Right X |
| ar_y | 8000 | ADXL Right Y |
| ar_z | 12000 | ADXL Right Z |
| bg_x | 5000 | Bogie X |
| bg_y | 4000 | Bogie Y |
| bg_z | 6000 | Bogie Z |

Alert condition: alertGenerated is true only when the 50 m window contains speed >= 80 kmph and at least one configured axis peak threshold is crossed. The cloud UI shows these alerts on a map and in a notification banner.

## 5.3 Fault Record: faults/faults.bin

| Offset | Field | Type | Meaning |
| --- | --- | --- | --- |
| 0 | timestampMs | uint64_t | Fault timestamp. Some fault sources use steady_clock, so treat as relative order unless source is upload failure. |
| 8 | faultCode | uint8_t | Fault code such as node timeout, upload failed, segment invalid, count jump, all vibration missing. |
| 9 | nodeId | uint8_t | Origin node: 1 AL, 2 AR, 3 bogie, 4 encoder, 0 gateway/internal. |
| 10 | severity | uint8_t | 1 warning, 2 error. |
| 11 | description | char[64] | Null-terminated ASCII fault description. |

## 5.4 Raw Packet File Format: raw/*.bin

| Offset | Field | Type | Meaning |
| --- | --- | --- | --- |
| 0 | packet_length | uint32_t | Byte count of following raw frame. |
| 4 | raw_frame | uint8_t[] | Complete raw packet including SOF, type, node, sequence, payload, CRC and EOF. |

# 6. Real-Time Alert JSON Contract

Endpoint: POST /api/v1/alerts. This endpoint is ready in the cloud app. The ICD notes that gateway HTTP POST for alert upload is planned/stubbed in the current gateway implementation, while PeakRecord50m.alertGenerated remains the authoritative trigger.

| Field | JSON Type | Unit | Meaning |
| --- | --- | --- | --- |
| gatewayId | string | - | Gateway logical ID. |
| trainId | string | - | Train identifier. |
| sessionName | string | - | Active session name. |
| windowStartMm | integer | mm | Start of 50 m peak window. |
| windowEndMm | integer | mm | End of 50 m peak window. |
| speedKmph | float | km/h | Maximum speed in window. |
| triggeredAxes | array<object> | - | One object per axis exceeding threshold. |

| triggeredAxes Field | JSON Type | Unit | Meaning |
| --- | --- | --- | --- |
| axisName | string | - | Axis name such as al_x, ar_y, bg_z. |
| peakValueMg | integer | mg | Measured peak value. |
| thresholdMg | integer | mg | Threshold crossed by that axis. |
| peakPositionMm | integer | mm | Track position of peak. |
| peakLat | float | deg | Latitude for map marker. |
| peakLon | float | deg | Longitude for map marker. |

# 7. What the Cloud Processes and Stores

Extract gatewayId, trainId and sessionName from ZIP filename.

Open ZIP and verify required files are present.

Parse session_metadata.json and check schemaVersion = 1.0.

Validate filename identifiers against metadata identifiers.

Check duplicate key (gatewayId, sessionName) for idempotency.

Check fixed binary file sizes: RMS % 66, Peak % 302, Fault % 75.

Parse RMS, Peak, Fault and Raw packet metadata.

Store original ZIP, extracted files and parsed records in PostgreSQL.

Create alert events when Peak alertGenerated is true and speed >= 80 kmph.

Expose CSV downloads and dashboards for verification and reporting.

| Database Table | Important Columns | Purpose |
| --- | --- | --- |
| archives | archive_id, gateway_id, train_id, session_name, archive_name, checksum, validation_status | Root record for each uploaded ZIP archive. |
| sessions | session_id, archive_id, gateway_id, train_id, session_status, created_utc | Logical session metadata parsed from JSON. |
| extracted_files | file_relative_path, file_size_bytes, storage_uri, integrity_ok, file_bytes | Stored extracted binary files and integrity result. |
| rms_records | position_mm, latitude, longitude, valid_mask, al/ar/bg mg fields | Spatial acceleration data per about 25 cm segment. |
| peak_records | window_start_mm, window_end_mm, speed_kmph, alert_generated, axis_data_json | Processed peak data over 50 m windows. |
| fault_records | timestamp_ms, fault_code, node_id, severity, description | Gateway/sensor fault events. |
| raw_packet_records | file_relative_path, packet_length, sof, packet_type, node_id, sequence_number | Metadata for raw forensic packets. |
| cloud_alert_events | gateway_id, train_id, session_name, speed_kmph, triggered_axes_count, payload_json | Real-time/map alert records. |

# 8. API Contract

| Method | Endpoint | Payload / Body | Response / Purpose |
| --- | --- | --- | --- |
| PUT | /api/v1/archive | Raw ZIP bytes; filename query/header | 201 Created for new archive; 200 for duplicate; 422 for mismatch. |
| GET | /api/v1/archives | limit query | JSON list of uploaded archives. |
| GET | /api/v1/archives/{archive_id}/files | archive_id | JSON list of extracted files for one archive. |
| POST | /api/v1/alerts | CloudAlertEvent JSON | 200 OK; stores alert event for map/notification. |
| GET | /api/v1/alerts | limit query | JSON alert events with map-ready latitude/longitude. |
| GET | /csv/reports | none | CSV report metadata. |
| GET | /csv/preview/{name} | report name | JSON preview of selected report. |
| GET | /csv/download/{name} | report name | CSV export file. |

# 9. What the Cloud Sends Back to the Gateway

| Condition | HTTP Status | Gateway Meaning |
| --- | --- | --- |
| Archive stored and parsed successfully | 201 Created | Gateway can delete local ZIP and session folder. |
| Archive already exists by (gatewayId, sessionName) | 200 OK | Idempotent success; gateway can stop retrying and delete local copy. |
| Malformed ZIP or missing upload body | 400 Bad Request | Gateway should treat as failed upload and retain/retry. |
| Filename does not match session_metadata.json | 422 Unprocessable Entity | Validation rejection; gateway should retain local copy for diagnostics/retry. |
| Database/storage/server error | 5xx | Gateway retries based on upload retry policy. |
| Real-time alert JSON accepted | 200 OK | Alert event stored for notification/map. |

Cloud-to-gateway data: In the current frozen interface, the cloud does not yet send route files or remote configuration back to the gateway. The only implemented return path is HTTP status/JSON response for archive and alert requests. Remote route/config endpoints are listed as future/not implemented in the interface document.

# 10. UI and Demonstration Pages

| UI Page / Tab | What It Demonstrates |
| --- | --- |
| Gateway Archive | Upload ICD ZIP archive from PyQt desktop app. Shows stored archives after upload. |
| Parsed Records | Preview parsed archive records: archives, RMS, peak, fault and raw packet metadata. |
| Alert Events | Shows alert event rows received or derived from peak records. |
| Cloud Summary | Shows API/database status and archive/CSV endpoints. |
| /cloud-dashboard | Public web dashboard with upload, data contract summary, CSV links and parsed tables. |
| /csv-page | Upload archive, preview reports and download CSV exports. |
| /alerts-page | Map visualization and notification banner when alert speed is >= 80 kmph. |

# 11. Current Implementation Status

| Requirement | Status | Evidence / Notes |
| --- | --- | --- |
| ZIP upload endpoint | Done | PUT /api/v1/archive implemented; stores and parses archive. |
| Required archive structure | Done | Validates session_metadata.json, RMS, peak, fault and raw files. |
| Database storage | Done | PostgreSQL tables for archives, sessions, extracted files and parsed records. |
| CSV reports | Done | CSV preview/download endpoints and web page. |
| Alert map/notification | Done | Alert page shows map markers and notification banner for speed-gated alerts. |
| Real gateway alert POST | Cloud ready; gateway pending | Cloud endpoint is ready. ICD says gateway HTTP POST alert upload is still a future/stub area. |
| Remote route/config sent to gateway | Not implemented | ICD marks cloud route/config endpoint as future/not yet implemented. |
| ADXL conversion confirmation | Pending firmware confirmation | ICD notes ADXL factor has TODO in gateway source. |

# Appendix A: Example Archive Name and Folder

Archive name example: GW_BOGIE_001__TRAIN_07__SESSION_20260609_083015.zip

session_metadata.json
rms/rms_25cm.bin
peak/peak_50m.bin
faults/faults.bin
raw/adxl_left.bin
raw/adxl_right.bin
raw/bogie.bin
raw/encoder.bin